X-AXIS FIRST TEST - PHASE-BY-PHASE COMMISSIONING PROCEDURE

KOTEC KTPG-320 | UC300 (EC500) + Mach3 | X axis = YPV drive n1 | SOMIK Skikda

| Machine: | KOTEC KTPG-320 CNC Plasma Cutting Machine |
| --- | --- |
| Axis under test: | X (gantry long travel) - left / right jog |
| Drive: | YPV n1 - YBL13S-75L.Z (1000 W, 5.5 A, 2000 rpm, encoder 8P / 2500) |
| Controller: | Digital Dream UC300 (EC500) - USB + Mach3 + DigitalDream plugin |
| Ballscrew pitch (X): | 10 mm / rev |
| Steps per mm (X): | 1000 (ELGN = ELGD = 10000) |
| Jog velocity: | 2000 mm/min |
| Jog acceleration: | 300 mm/s2 |
| Date: |  |
| Technician: |  |
| Result: | Pass / Fail |

SAFETY: the plasma torch must remain OFF and disconnected from gas and power throughout all phases. Keep an extinguisher nearby and stay clear of the moving gantry. Cut mains + both 24V supplies before any wiring.

PHASE A - PREPARE (POWER OFF)

Cut the electrical cabinet mains (220/380 V AC) and BOTH 24V DC supplies (System + IO).

Verify with a multimeter (AC volts): 0 V between R-S, R-T, S-T on YPV drive n1.

Confirm the charge LED on drive n1 is fully OFF before touching any terminal.

Never touch terminals R, S, T, U, V, W while the charge LED is lit.

Identify hardware: X drive = YPV n1, feeding the X motor YBL13S-75L.Z (1000 W, 5.5 A).

Gather material: shielded twisted-pair cable (STEP/DIR, max 3 m), 2x 24V relays (coil < 100 mA), NC E-stop button, 24V PSU(s) >= 1 A (System + IO), screwdrivers, multimeter, wire strippers.

PHASE A - CHECKLIST

☐  Mains + both 24V supplies OFF

☐  R/S/T on drive n1 reads 0 V (multimeter)

☐  Charge LED on drive n1 fully OFF

☐  X drive = YPV n1 and X motor identified

☐  Material available (cable, relays, E-stop, PSUs, tools)

PHASE B - WIRE X ONLY (POWER STILL OFF)

B.1 - Power wiring

Mains -> YPV drive n1 power terminals.

24V DC System (GND, 24V) -> UC300 System terminal block (>= 1 A).

24V DC IO (COM-, COM+) -> UC300 IO terminal block (>= 1 A).

Drive logic power: 24G on CN1-5 (I/O common 24V).

B.2 - STEP / DIR from UC300 to drive (CN1)

Wire COM- (UC300) -> DGND (CN1-31) FIRST - reference for all other signals (mandatory).

XP+ -> CN1-26 (+PPCMD)  /  XP- -> CN1-27 (-PPCMD)   [STEP]

XD+ -> CN1-28 (+NPCMD)  /  XD- -> CN1-29 (-NPCMD)   [DIR]

Use shielded twisted-pair cable; separate pair for STEP and DIR.

Ground the cable shield ONLY at the UC300 cabinet side (never both ends).

B.3 - SON / RST relay circuits

Never connect a UC300 output directly to CN1-6 or CN1-7 - a 24V relay is mandatory. Relay coil current must be < 100 mA (ULN2803A internal driver limit).

SON relay - coil circuit (UC300 side): +24V DC -> coil pin 1 ... coil pin 2 -> OT1 (Spindle terminal, pin 3).

SON relay - contact circuit (drive side, use NO contact): CN1-6 (/SON) -> COM ... NO -> CN1-5 (24G).

RST relay - coil circuit (UC300 side): +24V DC -> coil pin 1 ... coil pin 2 -> OUT4 (DB9 General Output, pin 2).

RST relay - contact circuit (drive side, use NO contact): CN1-7 (/RST) -> COM ... NO -> CN1-5 (24G).

B.4 - Motor + safety

Motor power U / V / W between drive n1 and X motor (cable rated >= 5.5 A, recommended 8-10 A).

E-Stop button wired NC (normally closed) -> IN01. Never wire it NO.

B.5 - Continuity check (power off)

With multimeter continuity: verify CN1-5 (24G), CN1-6 (/SON), CN1-7 (/RST), CN1-26/27 (+PPCMD/-PPCMD), CN1-28/29 (+NPCMD/-NPCMD), CN1-31 (DGND).

Confirm no continuity between COM- (UC300) and any motor power terminal (U/V/W).

PHASE B - CHECKLIST

☐  Mains -> drive n1; 24V System + IO -> UC300; 24G -> CN1-5

☐  COM- / DGND -> CN1-31 wired first (mandatory)

☐  XP+/XP- -> CN1-26/27 (STEP); XD+/XD- -> CN1-28/29 (DIR)

☐  Shield grounded at UC300 cabinet side only

☐  SON relay: OT1 -> coil; CN1-6 / NO / CN1-5

☐  RST relay: OUT4 -> coil; CN1-7 / NO / CN1-5

☐  Coil current of both relays < 100 mA

☐  Motor U/V/W connected; E-Stop NC -> IN01

☐  Continuity verified on CN1 pins; no short to U/V/W

PHASE C - YPV SETUP (X DRIVE ONLY, CN3 / RS-232)

Use JS YPV Setup software connected to CN3 (RS-232) of drive n1. These parameters are independent of the controller.

| Reg. | Parameter | Value | Description | Priority |
| --- | --- | --- | --- | --- |
| #1 | CNTL | 2 | Control mode = Position (P mode) | CRITICAL |
| #2 | PMOD | 0 | Pulse format = PULSE/DIR | CRITICAL |
| #32 | ELGN | 10000 | Electronic gear numerator | CRITICAL |
| #36 | ELGD | 10000 | Electronic gear denominator (steps/mm = 1000) | CRITICAL |
| #44 | RPM | 2000 | Motor rated speed (nameplate) | IMPORTANT |
| #45 | ENCO | 2500 | Encoder resolution (points/rev) | IMPORTANT |
| #43 | MTYP | 8 | Motor pole count (8P) | IMPORTANT |
| #7 | PACC | 0 | Accel/decel time (Mach3 handles the ramp) | IMPORTANT |
| #53 | PERZ | 5000 | Following-error limit | USEFUL |
| #59 | MRO | 1 | Rotation direction (1=CW, 0=CCW) - verify on test | USEFUL |

Open JS YPV Setup and connect to drive n1 via CN3.

Enter the values from the table above.

CLICK 'Upload and Program' to write to FLASH ROM - otherwise settings are lost on power off.

PHASE C - CHECKLIST

☐  CNTL = 2 (Position mode)

☐  PMOD = 0 (PULSE/DIR)

☐  ELGN = 10000 / ELGD = 10000

☐  RPM = 2000 / ENCO = 2500 / MTYP = 8 / PACC = 0

☐  PERZ = 5000 / MRO = 1

☐  'Upload and Program' executed (FLASH ROM)

PHASE D - MACH3 + PLUGIN SETUP (PC)

D.1 - Install plugin and create profile

Copy DigitalDream.dll into C:\Mach3\PlugIns.

Launch Mach3. In 'Session Profile' click 'Create Profile', name it 'DigitalDream', check 'Default Profile Values', OK.

In the hardware selection window choose 'DigitalDream-DigitalDream-PlugIn' (NOT 'Normal Printer port').

Verify the message 'UC300 device is connected to your computer'.

Config > Config Plugins: confirm DigitalDream-PlugIn is checked (enabled).

D.2 - Motor Outputs (Config > Ports & Pins > Motor Outputs)

X axis: check 'Enabled'. Step Pin# = 2, Dir Pin# = 3 (Port No. 2).

Click Apply. Do NOT check Dir LowActive yet (adjust after direction test).

Click 'Save Axis Settings'.

D.3 - Output Signals (Config > Ports & Pins > Output Signals)

Map OT1 / OUT1 (Spindle port, pin 3) to the output used for X SON - drive it via Mach3 spindle enable (M3) or an Output Signal on Port 2, pin 1.

Map OUT4 (DB9 pin 2) to RST X - Port 2, pin 4.

UC300 outputs are defined on Port No. 2 by default. Keep wiring and Mach3 mapping in sync.

D.4 - Input Signals (Config > Ports & Pins > Input Signals)

Map E-Stop -> IN01 (Port 2, pin 1). Check 'Emulated' if required by the plugin version.

D.5 - Motor Tuning (Config > Motor Tuning)

Select X axis. Steps per Unit = 1000. Velocity = 2000 mm/min. Acceleration = 300 mm/s2.

Click 'SAVE AXIS SETTINGS' before moving on.

PHASE D - CHECKLIST

☐  DigitalDream.dll installed in C:\Mach3\PlugIns

☐  Profile 'DigitalDream' created with DigitalDream-PlugIn

☐  UC300 'device connected' message shown

☐  X Motor Output: Step 2 / Dir 3, Port 2, Enabled

☐  SON X mapped to OT1/OUT1 (port 2 pin 1)

☐  RST X mapped to OUT4 (port 2 pin 4)

☐  E-Stop mapped to IN01

☐  Motor Tuning X: 1000 steps/unit, 2000 mm/min, 300 mm/s2 - saved

PHASE E - POWER UP AND FIRST JOG

Torch still OFF gas + power. Move slowly. Keep hands clear of the gantry.

Power up 24V System, then 24V IO, then mains. Confirm the charge LED lights on drive n1.

In Mach3 confirm the UC300 connection is active ('device is connected').

TEST E-STOP: press the E-stop button - an alarm must appear immediately in Mach3. Reset it.

Enable X SON (turn on spindle / SON X output): drive n1 must switch to 'Servo ON'.

If the drive does not enable: check relay coil circuit, DGND / COM- on CN1-31, continuity CN1-6 <-> CN1-5, and coil current < 100 mA.

Jog X a few mm at low feed: jog LEFT then jog RIGHT (Tab jog or arrow keys).

Check the direction matches the command.

If reversed: correct ONLY ONE setting - either MRO in YPV Setup or Dir LowActive in Mach3 - never both at the same time. Save Axis Settings after any change.

Verify both left and right movement at low speed, then gradually increase to the target jog speed.

PHASE E - CHECKLIST

☐  Power up sequence OK (24V System -> IO -> mains)

☐  Charge LED on drive n1 lit

☐  UC300 connected in Mach3

☐  E-Stop tested (alarm appears / machine stops)

☐  X drive in 'Servo ON' when SON enabled

☐  X jog LEFT works

☐  X jog RIGHT works

☐  Direction correct (or corrected with only ONE setting)

☐  No alarm / following-error during jog

PHASE F - CALIBRATION SANITY (OPTIONAL)

In Mach3 jog X a known distance, e.g. 100 mm.

Measure the actual travel with a tape measure.

If error: new ELGD = old ELGD x (measured distance / commanded distance).

Reprogram ELGD in YPV Setup AND update Steps per Unit in Mach3 Motor Tuning (both must stay consistent). 'Upload and Program' the drive.

Repeat until the error is < 0.1 mm on 100 mm.

PHASE F - CHECKLIST

☐  100 mm jog performed

☐  Measured travel recorded: ______ mm

☐  ELGD / Steps per Unit corrected if needed

☐  Error < 0.1 mm on 100 mm

RESULT

| Result | Signature |
| --- | --- |
| Pass / Fail |  |