ELECTRIC PLAN - CNC PLASMA MACHINE RETROFIT

KOTEC KTPG-320 | Controller: Digital Dream UC300 (EC500) + Mach3 | SOMIK Skikda

| Client: | SOMIK Skikda - Skikda |
| --- | --- |
| Machine: | KOTEC KTPG-320 CNC Plasma Cutting Machine |
| Serial No: | 149P324050WYPV1K |
| Motion Controller: | Digital Dream UC300 (EC500) - USB |
| Software: | Mach3 + DigitalDream Plugin (Digitaldream20200905.dll) |
| X / Y axes: | YPV Series AC servo drives (YE LI YBL13S-75L.Z) |
| Z axis: | Retained SLO-SYN stepper + EPC encoder + new DM556 driver |
| Reference: | Guide_Retrofit_UC300_CNC_Plasma.pdf (SEDDARI SMART SYSTEMS) |
| Date: | 20/07/2026 |

1. UC300 (EC500) CONTROLLER OVERVIEW

| Feature | Specification |
| --- | --- |
| PC connection | USB (shielded cable recommended) |
| Motion axes | 3-6 axes (X, Y, Z, A, B, C) - pulse/direction |
| Step frequency | Up to 300 kHz per axis |
| Digital inputs | 12 opto-isolated inputs (IN01 - IN12) |
| Digital outputs | 10 opto-isolated outputs (OUT1 - OUT10) with ULN2803A relay driver |
| Spindle port | GNDs, VSO (0-10V), OT1 (OUT1), OT2 (OUT2), Index |
| MPG port | Standard MPG or Digital Dream NV-MPG (encoder A/B inputs) |
| Power | System 24V DC >= 1 A + IO 24V DC >= 1 A (common GND) |

Motor output pins are FIXED by the controller board. In Mach3 all outputs are assigned to Port No. 2. The general output driver ULN2803A can switch relay coils up to 100 mA.

2. MOTOR AXES - STEP/DIR WIRING TO YPV DRIVES (CN1)

| Axis | UC300 | YPV CN1 | Function |
| --- | --- | --- | --- |
| X | XP+ | CN1-26 | +PPCMD (STEP+) |
| X | XP- | CN1-27 | -PPCMD (STEP-) |
| X | XD+ | CN1-28 | +NPCMD (DIR+) |
| X | XD- | CN1-29 | -NPCMD (DIR-) |
| X | COM- | CN1-31 | DGND - common logic (MANDATORY) |
| Y | YP+ / YP- | CN1-26 / 27 | +PPCMD / -PPCMD |
| Y | YD+ / YD- | CN1-28 / 29 | +NPCMD / -NPCMD |
| Y | COM- | CN1-31 | DGND - common logic (MANDATORY) |
| Z | ZP+ / ZP- | -> THC unit -> DM556 | STEP+ / STEP- |
| Z | ZD+ / ZD- | -> THC unit -> DM556 | DIR+ / DIR- |

Wire the COM- (UC300) to DGND (CN1-31) FIRST on each drive - it is the reference for all other signals.

Use shielded twisted-pair cable for STEP and DIR (separate pairs), maximum 3 m per axis.

Connect the cable shield ONLY at the UC300 cabinet side (never both ends).

YPV outputs are 5V TTL; YPV drive inputs are opto-isolated (no breakout board needed).

3. Z-AXIS RETROFIT

Original controller cabinet and Z-axis controller are missing.

Retained hardware: Superior Electric SLO-SYN stepper motor + EPC encoder.

New DM556 digital stepper driver added to drive the SLO-SYN motor.

Standalone HF-rated THC unit (Proma Compact THC SD) installed between UC300 and DM556.

THC acts as an electrical barrier protecting the UC300 from HF ignition (up to 17 kV).

EPC encoder (A/B) wired to UC300 fast inputs (MPG port) for position monitoring and crash detection.

Signal chain: UC300 ZP/ZD -> THC (pass-through) -> DM556 -> SLO-SYN motor.

4. DRIVE ENABLE - SON / RST RELAYS

YPV drives require servo-enable (/SON, CN1-6) and reset (/RST, CN1-7). A 24V relay is mandatory between UC300 and the drive - NEVER connect a UC300 output directly to CN1-6 or CN1-7.

| Function | UC300 Output | Location | YPV Target |
| --- | --- | --- | --- |
| X SON | OT1 | Spindle port, pin 3 | YPV X - CN1-6 (/SON) |
| Y SON | OT2 | Spindle port, pin 4 | YPV Y - CN1-6 (/SON) |
| Z EN | OUT3 | DB9, pin 6 | DM556 EN+ (optional) |
| X RST | OUT4 | DB9, pin 2 | YPV X - CN1-7 (/RST) |
| Y RST | OUT5 | DB9, pin 7 | YPV Y - CN1-7 (/RST) |
| Spare | OUT6 | DB9, pin 3 | Spare (RST Z if YPV added) |

Relay coil circuit (UC300 side): +24V DC -> relay coil -> UC300 output. Relay contact (drive side, use NO contact): CN1-6 or CN1-7 -> relay COM ... relay NO -> CN1-5 (24G). Relay coil current must be < 100 mA (ULN2803A limit). If higher, add an external driver stage.

OT1/OT2 are physically on the Spindle terminal block but are generic opto-isolated outputs - they can be reused for SON exactly like OUT3-OUT10.

5. SPINDLE CONTROL PORT (OT1 / OT2)

| Terminal | Signal | Use |
| --- | --- | --- |
| 1 | GNDs | Output ground |
| 2 | VSO | 0-10V analog out - spare |
| 3 | OT1 (OUT1) | SON X relay -> YPV X CN1-6 |
| 4 | OT2 (OUT2) | SON Y relay -> YPV Y CN1-6 |
| 5 | Index | Speed feedback input - spare |

6. GENERAL OUTPUT INTERFACE (DB9) - OUT3 TO OUT10

| DB9 Pin | Signal | Assignment |
| --- | --- | --- |
| 1 | GND (COM-) | Common ground for outputs |
| 2 | OUT4 | RST X relay -> YPV X CN1-7 |
| 3 | OUT6 | Spare |
| 4 | OUT8 | Spare (gas solenoid - gas console is manual) |
| 5 | OUT10 | Spare |
| 6 | OUT3 | Z EN -> DM556 EN+ |
| 7 | OUT5 | RST Y relay -> YPV Y CN1-7 |
| 8 | OUT7 | Arc-On (plasma start) -> ignition console |
| 9 | OUT9 | HF Ignition start -> ignition console |

DB9 pin order per the retrofit guide (Picture 5-5 of the UC300 manual). Verify the pin order on the physical connector before wiring.

7. INPUT PORT (IN01 - IN12)

| Input | Signal | Connected To |
| --- | --- | --- |
| IN01 | E-Stop (NC) | Emergency stop button - NC contact |
| IN02 | Home X | X home sensor (NPN 3-wire) |
| IN03 | Home Y | Y home sensor (NPN 3-wire) |
| IN04 | Home Z | Z home sensor (NPN 3-wire) |
| IN05 / IN06 | Limit X+ / X- | X limit switches |
| IN07 / IN08 | Limit Y+ / Y- | Y limit switches |
| IN09 / IN10 | Limit Z+ / Z- | Z limit switches |
| IN11 | Spare | Reserved: THC UP (if relay-type THC) |
| IN12 | Spare | Reserved: THC DOWN / Arc OK |

E-Stop MUST be wired NC (normally closed) on IN01 - never NO.

NPN 3-wire sensor color code: BROWN = COM+ | BLACK = input channel | BLUE = COM-.

2-wire switches connect between COM- and the input channel.

All 12 inputs are used/allocated. The HPR Arc-OK signal is NOT wired separately in this configuration (use IN11/IN12 later if needed).

8. EPC ENCODER FEEDBACK (MPG PORT)

| MPG Signal | Encoder Connection | Use |
| --- | --- | --- |
| WHA+ / WHA- | EPC encoder A / A- | Position feedback (fast input) |
| WHB+ / WHB- | EPC encoder B / B- | Quadrature position feedback |
| +5V / GND | Encoder supply (if within current) | Encoder power |
| XIN / YIN / ZIN | - | Axis select (MPG) - not used |
| EP | - | MPG E-stop - not used |

Set the MPG function switch on the controller to STD before using the port for the encoder. Configure as Encoder/MPG in Mach3: Config > Ports & Pins > Encoder/MPG's.

9. POWER SUPPLY (SYSTEM + IO)

| Terminal block | Terminals | Function |
| --- | --- | --- |
| System | GND, 24V | Power for the UC300 board itself (>= 1 A) |
| IO | COM-, COM+ | Power for the input/output ports (>= 1 A) |
| Servo drives | 220V AC + CN1 | YPV drives X and Y (power + logic) |
| Stepper driver | 24-50V DC | DM556 (Z axis) |
| THC unit | 7-35V DC (same 24V rail) | Proma Compact THC SD |

System and IO share one internal common GND. Use a single 24V PSU sized for both (or two PSUs tied to the same ground). LEDS: power + IO power (red when OK), CONN lit when Mach3 is linked.

10. YPV DRIVE PARAMETERS (JS YPV Setup, CN3 / RS-232)

| Reg. | Parameter | Value | Description | Priority |
| --- | --- | --- | --- | --- |
| #1 | CNTL | 2 | Control mode = Position (P mode) | CRITICAL |
| #2 | PMOD | 0 | Pulse format = PULSE/DIR | CRITICAL |
| #32 | ELGN | calculate | Electronic gear numerator | CRITICAL |
| #36 | ELGD | calculate | Electronic gear denominator | CRITICAL |
| #44 | RPM | 2000 | Motor rated speed (plaque) | IMPORTANT |
| #45 | ENCO | 2500 | Encoder resolution (points/rev) | IMPORTANT |
| #43 | MTYP | 8 | Motor pole count (8P) | IMPORTANT |
| #7 | PACC | 0 | Accel/decel time (Mach3 handles ramp) | IMPORTANT |
| #53 | PERZ | 5000 | Following-error limit | USEFUL |
| #59 | MRO | check | Rotation direction (1=CW, 0=CCW) | USEFUL |

Steps/mm in Mach3 = (ENCO x 4 x ELGD / ELGN) / leadscrew pitch (mm/rev). After any change use 'Upload and Program' in YPV Setup to write to FLASH ROM.

11. HF PLASMA SAFETY NOTES (17 kV)

The Hypertherm console uses high-frequency ignition (up to 17 kV).

The standalone THC unit (Proma Compact THC SD) is the electrical barrier between the UC300 and the plasma arc.

Arc voltage is measured through the THC built-in divider (1:50) - never feed arc voltage directly to the UC300.

Ground the ignition console, gas console and workpiece to a common earth point.

Keep torch leads and HF wiring physically separated from all signal and encoder wiring.

Use shielded cables for encoder (EPC -> UC300) and step/dir lines; ground shields at one end only.

12. MACH3 CONFIGURATION

12.1 - Plugin installation

Copy DigitalDream.dll into C:\Mach3\PlugIns.

Launch Mach3, Create Profile 'DigitalDream', select 'DigitalDream-DigitalDream-PlugIn'.

Verify the status bar shows 'UC300 device is connected to your computer'.

Config > Config Plugins: check DigitalDream-PlugIn is enabled.

12.2 - Motor Outputs (Config > Ports & Pins > Motor Outputs)

| Axis | Step Pin# | Dir Pin# | Dir LowActive |
| --- | --- | --- | --- |
| X | 2 | 3 | per actual direction |
| Y | 4 | 5 | per actual direction |
| Z | 6 | 7 | check if Z inverted (Up=+) |

12.3 - Input Signals

Map per Section 7: E-Stop -> IN01, Home X/Y/Z -> IN02/03/04, Limits -> IN05..IN10, THC -> IN11/12. Check 'Emulated' if required by the plugin version.

12.4 - Output Signals

All UC300 outputs are on Port No. 2. Map per Sections 4/6: OT1/OT2/OUT3 -> SON/EN, OUT4/OUT5 -> RST, OUT7 -> Arc-On, OUT9 -> HF Ignition.

12.5 - Motor Tuning (Config > Motor Tuning)

Steps per Unit: same value as used for ELGN/ELGD in YPV (must stay consistent).

Velocity and Acceleration: set to avoid stalling the servo.

Click SAVE AXIS SETTINGS for EACH axis before moving to the next.

13. CALIBRATION - STEPS/MM

Move axis a known distance (e.g. 100 mm) and measure actual travel.

If error: new ELGD = old ELGD x (measured distance / commanded distance).

Reprogram ELGD in YPV Setup AND Steps per Unit in Mach3 (both must stay consistent).

Repeat until error < 0.1 mm on 100 mm. Repeat for Y and Z.

14. WIRING DIAGRAM

Wiring diagram is also available as separate file: Wiring_Diagram.png (lines + dots style).

15. FINAL CHECKLIST (BEFORE POWER-ON)

COM- (UC300) / DGND (CN1-31) wired on both YPV drives.

Shields of STEP/DIR and USB cables grounded at cabinet side only.

E-Stop wired NC on IN01 and tested.

NPN sensors wired with correct color code (Brown=COM+ / Black=IN / Blue=COM-).

SON/RST relays mounted and tested; coil current < 100 mA verified.

All 10 outputs allocated per Sections 4/6 and noted for reference.

YPV critical parameters (CNTL, PMOD, ELGN, ELGD) programmed and saved.

DigitalDream plugin installed; UC300 recognized in Mach3.

Motor Outputs / Input Signals / Output Signals / Motor Tuning configured and saved.

Steps/mm calibrated on each axis; full Home cycle done without error.

Dry arc-ignition test done, then test cut on scrap plate.

16. CN3 COMMUNICATION PORT (YPV DRIVE - YPV SETUP PROGRAMMING)

CN3 is the YPV drive's RS-232C (DB9, 9-pin D-Type) communication port, used with the 'YPV Setup' software (YE LI icon) to read/write drive parameters (CNTL, PMOD, ELGN/ELGD, ENCO, RPM, ...). Source: YE-LI Electric & Machinery manual, Appendix 1 'YPV RS232-USB' + Driver Configuration chapter (Vol 1-7).

| CN3 (DB9) Pin | Signal | Wire (factory cable) | To |
| --- | --- | --- | --- |
| 2 | TX (RS-232 transmit) | White | USB data line (adapter RX) |
| 3 | RX (RS-232 receive) | Green | USB data line (adapter TX) |
| 5 | GND (signal ground) | Black | USB GND |
| 4 | VCC (power) | - | USB 5V (optional supply) |

Only pins 2, 3, 5 are required for basic communication; add pin 4 only if the adapter needs external 5V.

TX/RX are crossed (null-modem style) between the drive and a DTE PC - confirm orientation on the actual cable.

Install 'YPV Setup' (VER7501 for VER0501 hardware; unmarked VER2 = 031220 -> program 031220). Remove older versions first.

Backup the default configuration file BEFORE changing any parameter.

After editing a parameter: Enter -> Burn -> wait ~5s -> Reboot to apply.

The 7.5 kW connection diagram shows CN3 as a 9-pin RS-232 feed to the Parameter Setting Terminal next to CN1/CN2.

17. REFERENCE DOCUMENT

Guide_Retrofit_UC300_CNC_Plasma.pdf - Guide complet de remise en marche CNC Plasma KTPG-320 (UC300), SEDDARI SMART SYSTEMS (included in the Electric Plane folder).